Why an extra Shim ?

Why an extra shim (or what’s the name) for a real well designed final with rail cooling?

To small contact area to rail, risk of bending the board when tightening the screw

to tight. I had a good love, when seeing in Net the cooling device with the copper piece

attached to the side.

I wanted the contact area increase beside the rail, and a sufficient heath transfer to the casing.

Beside it allows tightening the 3mm screw enough. It should be cheap, easy to make and install, also efficient.

The shim makes thermal contact to the board, rail and casing side and bottom.

There for a 3mm screw is a must, and no enlarging of the shim hole. That’s needed for being able to press the shim to the side. But the casing bottom hole should fit, if not, that could be enlarged, in order to touch the side. Use enough thermal compounds.

You can check it by placing the shim before install along side the corner, with the 3m screw inserted also passing trough the bottom hole. The shim should touch the side. The surface of the shim should be even to the rail with a size of 5mm. An easy check. The shim is so designed that the board with shim touching the side, still allows a good fit of Front and Back plates.

My thanks go to Sam who transferred my drawing to NC.

To install: Follow up my instructions. Maybe some one else has a better idea for a shim…

There is no need to change any thing on the present production, all is working well, the shim could be an add on.

One thing I would change is on the filter board the SMA Antenna connecter to one with 4 pins soldered. Why, in many shack SMA/BNC Adapters are used. Great torque.

Cheers es 73 Chris HB9BDM