HL2 Shim Mounting Instruction for 40mm Enclosure

Chris HB9BDM

Step 1: Place the Hermes and Filter board in correct position and drill then a       3.5mm hole, according to former instructions.

Step 2: Use sandpaper to grind of the coating at the bottom of the enclosure,

also the shoulder part of the rail. This will assure good thermal contact.

Then coat the places with Thermal compound, also the Rail Shoulder and side.

Step3: Pic1, Positioning the enclosure accordingly.

Pic1

Step 4: Pic2, Check thickness of the shim, it should be 5.00mm or 0.1mm less.

Coat the board surface where the shim is placed also with Thermal compound.

Place the shim between board and enclosure, so it still sticks out (Pic1)

Narrower side of shim facing board, wider side of shim facing enclosure.

Turn the enclosure together with the board, shim in-between carefully around

(Pic2) and use the 3mm mounting screw to hold shim in place during the sliding in procedure, till you reach the mounting whole position.

Pic2

Step 5: Pic3 carefully remove the mounting screw and insert form the bottom of the enclosure. Secure with washer and nut, but don’t tighten.

Pic3

Step 6: Connect Hermes and Filter Boards. Then press the whole assembly fully onto the side, to ensure a good thermal contact with the side of the enclosure. Then tighten the 3mm screw. That concludes the job.